VHF Packet Airmail routes from Walla Walla

02-17-11

144.97     WWU/College Place Gateway   WA7V-10

1.  Access using Handshake mode, direct from throughout WW Valley

2.  From more distant locations (especially to the west) access by:

Keyboard mode, type/select WA7V-7 in “connect to” line, click green button

Once connected to WA7V-7, use lower screen to type C WA7V-10 (space between “C” and “WA7V-10) then hit “enter” button on computer keyboard.

As soon as connection made with WA7V-10, switch to Handshake mode to pass traffic

145.01     Jeff’s Walla Walla Gateway  AL1Q-10

1.  Access direct from throughout WW Valley using Handshake mode,  in “connect to” line, type or select AL1Q-10, click green button

2.  If using low power and/or from poor access area, access via Handshake mode, using digi-peater.

a.)  In “connect to” line, type or select AL1Q-10 v AL1Q-7  (don’t have to type “via” or  “v”, but easier to 	see than a space between  the two callsign groups)

b.)   Remain in Handshake mode, and click green button

145.01    Spokane area Gateways:

1.  In KEYBOARD mode, Access WR7VHF-3  node by typing or selecting “WR7VHF-3 v AL1Q-7” in “connect to” (not necessary to type “via” or “v”, but easier to see than a space between the two callsign groups)

2.  Once connected to WR7VHF-3 node, do one of the following to connect to various gateways:

a.)  For W7GBU-10 gateway, in lower screen, type C W7GBU-10 (space between “C” and “W7GBU-10) and press “enter” on computer keyboard.  Once connected, switch to HANDSHAKE mode to pass traffic.

b.)  For WR7VHF-10 gateway, in lower screen, type EITHER WR7VHF-2, OR WR7VHF-4, and press “enter” on computer keyboard.  Once connected to THAT node, in lower screen, type C WR7VHF-10.

Once connected to WR7VHF-10 gateway, switch to Handshake mode to pass traffic.

145.01    Pullman gateway K7LL-10

1.  In Keyboard mode, type or select ”WR7VHF-3 v AL1Q-7”, and click green button

2.  Once connected to WR7VHF-3 node, in lower screen, type K7LL-10, and press “enter” on computer keyboard

3.  Once connected to K7LL-10, switch to handshake mode to pass traffic

145.53     North/Central Washington Gateways:

For NC7C-10 Ellensburg:

1.  In Keyboard mode, type or select N7HHU-8 in “connect to” line, and click green button

2.  Once connected to N7HHU-8 node, in lower screen, type C NC7C-10 (space after “C”) and press “enter” on computer keyboard

3.  Once connected to NC7C-10, switch to Handshake mode to pass traffic

For WB7WSP-10 Ephrata:

1.  In Keyboard mode, type or select N7HHU-8 in “connect to” line, click green button

2.  Once connected to N7HHU-8 node, in lower screen, type C WB7WSP-7 (space after “C”) and press “enter” on computer keyboard

3.  Once connected to WB7WSP-7 node, in lower screen, type C WB7WSP-10 (space after “C”) and press “enter” on computer keyboard

4.  Once connected to WB7WSP-10, switch to Handshake mode to pass traffic

* It is theoretically possible to use other routes and gateways as well, however from my location, I can’t get a solid enough connection to WA7V-7 to branch off from there, and I don’t have an adequate antenna and elevation to reach Hermiston or Pendleton gateways direct.